Factory Ressetting a Device

When to Complete a Factory Reset:

- Device Alert light is illuminated and will not reset with button press or power cycling

- Device Alert light does not illuminate upon power up

- If all other steps have been completed and device is still not syncing with the dashboard

How to Complete a Factory Reset:

Depending on outlet location this process may require 2 people

- Unplug the Device

- While unplugged press and hold the LED Button

- Continue holding the button and plug the device in

- After plugging in continue to hold the button for approx. 20 seconds until the light blinks rapidly

- Release the button and wait a brief moment until the light begins to blinks 1 time per second

- The device has now been Factory Reset and is ready for the Wi-Fi configuration process.

Saving Wi-Fi Configuration to Device

How to reconfigure and save the wireless information

*note if adding a new device to your property ignore step 1 as the new device should already be in pairing mode.

Step 1.

Press the LED Button and hold for 10-15 seconds.

When button is released the Red LED light should

be blinking 1 time per second and ready for pairing.

Step 2.

Using a Laptop/Mobile Device open the Wi-Fi settings

and find the list of available Wi-Fi Networks,

The Device ID should be listed in the network list available.

Connect to the Device Network from the available

Wi-Fi networks list the device will be named ` D5 [Serial-Number]`

*Please note you must remain connected

to the Device’s Wi-Fi to complete the pairing.

Step 3.

Next on the same Laptop/Mobile Device from Step 2,

Open a web browser and type the web address `192.168.0.1`

The page that loads will scan for a moment then list all the available

networks within range of your D5 Device.

Select the desired network and type in your Wi-Fi password if applicable,

then click the green Save Configuration